MATERIAL SAFETY DATA SHEET (MSDS)

3-CHLOROANILINE (META CHLORO ANILINE)

**** SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION ****

MSDS Name		:	3-Chloroaniline

Company Identification	: 	SARNA CHEMICALS PVT. LTD. (Unit II)

Plot No. 1708,A2/1715 ,  III Phase, G.I.D.C., Vapi- 396 195.

Gujarat State. INDIA

**** SECTION 2 - COMPOSITION, INFORMATION ON INGREDIENTS ****

CAS#  			:	108-42-9

Chemical Name		:	3-Chloroaniline

Purity (By GLC) 	:	99.50 % Min.

Hazard Symbols	: 	T N

Risk Phrases		: 	23/24/25 33 50/53

**** SECTION 3 - HAZARDS IDENTIFICATION ****

EMERGENCY OVERVIEW

Appearance:

Clear Pale yellow color liquid.

Caution!

May be harmful if absorbed through the skin. Light sensitive. Air sensitive. May cause methemoglobinemia. Causes eye and skin irritation. Causes digestive and respiratory tract irritation. May cause cancer based on animal studies. May cause skin eruptions and/or acne. Target Organs:

Blood. Potential Health Effects

Eye:

Causes eye irritation.

Skin:

Causes skin irritation. May be absorbed through the skin in harmful amounts. Produces acneiform eruptions.

Ingestion:

May cause irritation of the digestive tract. May cause methemoglobinemia, cyanosis, convulsions, and death. Methemoglobinemia is characterized by dizziness, drowsiness, headache, breath shortness, cyanosis with bluish skin, rapid heart rate and chocolate-brown colored blood. May cause skin eruptions and/or acne.

Inhalation:

May cause respiratory tract irritation. May cause effects similar to those described for ingestion.

Chronic:

May cause cancer according to animal studies.

**** SECTION 4 - FIRST AID MEASURES ****

Eyes:

Immediately flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower lids. Get medical aid immediately.

Skin:

Flush skin with plenty of soap and water for at least 15 minutes while removing contaminated clothing and shoes. Get medical aid if symptoms occur.

Ingestion:

If victim is conscious and alert, give 2-4 cupfuls of milk or water. Get medical aid immediately. Induce vomiting by giving one teaspoon of Syrup of Ipecac.

Inhalation:

Get medical aid immediately. Remove from exposure to fresh air immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen.

Notes to Physician:

Treat symptomatically and supportively.

**** SECTION 5 - FIRE FIGHTING MEASURES ****

General Information:

As in any fire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent), and full protective gear.

Extinguishing Media:

In case of fire, use water, dry chemical, chemical foam, or alcohol-resistant foam.

Flash Point			:	-10.4 oC

NFPA Rating			:	Not published.

Explosion Limits, Lower		:	Not available.

Upper				:	Not available.

**** SECTION 6 - ACCIDENTAL RELEASE MEASURES ****

General Information: Use proper personal protective equipment as indicated in Section 8.

Spills/Leaks:

Absorb spill with inert material, (e.g., dry sand or earth), then place into a chemical waste container.

**** SECTION 7 - HANDLING and STORAGE ****

Handling:

Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Use with adequate ventilation. Avoid contact with skin and eyes. Do not ingest or inhale.

Storage:

Poison room locked.

**** SECTION 8 - EXPOSURE CONTROLS, PERSONAL PROTECTION ****

Engineering Controls:

Use adequate ventilation to keep airborne concentrations low. Exposure Limits

Personal Protective Equipment

Eyes:

Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166.

Skin:

Wear appropriate gloves to prevent skin exposure.

Clothing:

Wear appropriate protective clothing to minimize contact with skin.

Respirators:

Follow the OSHA respirator regulations found in 29CFR 1910.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 149 approved respirator when necessary.

**** SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES ****

Physical State		:	 Liquid

Appearance		:	 clear yellow to pale yellow

Odor			:	 None reported.

PH			:	 Not available.

Vapor Pressure		:	 0.13 mbar @ 20

Vapor Density		:	 Not available.

Evaporation Rate	:	 Not available.

Viscosity		:	 Not available.

Boiling Point		:	 230.5 oC

Decomposition Temperature: 	> 300 deg C

Solubility		:	 practically insoluble in water, Soluble in most organic solvents

Specific Gravity/Density	:	 1.2150gms/cm3

Molecular Formula	:	 C6H6ClN

Molecular Weight	:	 127.57

**** SECTION 10 - STABILITY AND REACTIVITY ****

Chemical Stability:

Stable under normal temperatures and pressures.

Conditions to Avoid:

None reported.

Incompatibilities with Other Materials:

Acids, acid chlorides, acid anhydrides, strong oxidizing agents. Hazardous Decomposition Products: Hydrogen chloride, nitrogen oxides, carbon monoxide, carbon dioxide, nitrogen. Hazardous Polymerization: Has not been reported.

**** **** SECTION 11 – TOXICOLOGICAL INFORMATION ****

Acute effects, material is irritating to mucous membranes and upper, respiratory tract. May be harmful by inhalation, ingestion, or skin absorption. Causes eye and skin irritation. To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

**** **** SECTION 12 – ECOLOGICAL INFORMATION ****

Data not yet available.

**** **** SECTION 13 – DISPOSAL CONSIDERATIONS ****

Dissolve or mix the material with a combustible solvent and incinerator equipped with an afterburner and scrubber. Observe all federal, state and local environmental regulations.

**** **** SECTION 14 - TRANSPORT INFORMATION ****

UN No: 			: 	2019

Shipping Name		:	CHLOROANILINES, LIQUID

Hazard Class		:	6.1

Packing Group		:	 II

Packing Instruction	:	P001

IMDG Page No.		:	89

**** **** SECTION 15 - REGULATORY INFORMATION ****

Harmful by inhalation & if in contact with skin. Toxic to aquatic organisms & may cause long term adverse effects in the aquatic environment.

Wear suitable gloves & avoid release to the environment.

**** SECTION 16 - ADDITIONAL INFORMATION ****

MSDS Creation Date:  7/16/1996  Revision #0 Date: Original.

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitability of the information for their particular purposes. In no way shall Sarna be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if Sarna has been advised of the possibility of such damages.